ESP32 Connections:

| Component | Component Pin | ESP32 Pin |
| --- | --- | --- |
| DHT11 | VCC | 3.3V |
| DHT11 | GND | GND |
| DHT11 | DATA | GPIO 4 |
| MQ2 | VCC | VIN (5V) |
| MQ2 | GND | GND |
| MQ2 | AO | GPIO 34 |
| MQ2 | DO | Not connected |
| Buzzer | + (long leg) | GPIO 26 |
| Buzzer | - (short leg) | GND |
| I2C LCD | VCC | VIN (5V) |
| I2C LCD | GND | GND |
| I2C LCD | SDA | GPIO 21 |
| I2C LCD | SCL | GPIO 22 |
| Onboard LED | — | GPIO 2 (built-in) |

Pico2W Connections:

| Component | Component Pin | Pico2W Pin |
| --- | --- | --- |
| HC-SR04 | VCC | VBUS Pin 40 (5V) |
| HC-SR04 | GND | GND Pin 38 |
| HC-SR04 | TRIG | GP14 Pin 19 |
| HC-SR04 | ECHO | Voltage divider → GP15 Pin 20 |
| Resistor R1 | — | ECHO → R1 → GP15 |
| Resistor R2 | — | GP15 → R2 → GND |
| Onboard LED | — | Built-in |

HC-SR04 Voltage Divider (ECHO pin protection):

ECHO → [1.6kΩ R1] → GP15

↓

[1.6kΩ R2]

↓

GND

STM32 Blue Pill Connections:

| Component | Component Pin | Blue Pill Pin |
| --- | --- | --- |
| Button | Pin 1 | PB0 |
| Button | Pin 2 | GND |
| External LED | + long leg | PB1 |
| External LED | - short leg | GND via 1.6kΩ resistor |
| Onboard LED | — | PC13 (built-in, active LOW) |
| ST-Link | SWDIO | DIO |
| ST-Link | SWCLK | CLK |
| ST-Link | GND | GND |
| ST-Link | 3.3V | 3V3 |

STM32 → Pico2W UART Bridge:

Blue Pill PA9  (TX) → Pico2W GP1 (RX) Pin 2

Blue Pill PA10 (RX) → Pico2W GP0 (TX) Pin 1

Blue Pill GND       → Pico2W GND      Pin 3

Do NOT connect 3.3V between them — each board has its own power.

Power Sources:

| Device | Power |
| --- | --- |
| ESP32 | USB to PC/laptop |
| Pico2W | USB to powerbank or PC |
| STM32 | ST-Link 3.3V (via programming cable) |
| ST-Link | USB to PC |